Electric Motor Thermal Prediction and Cooling Advisory

Target variables, algorithms and methodology

18 October 2026

# 1. Purpose and scope

This report defines what the system must predict (the target variables), how each one is produced, and which algorithms are used. The data source is the Kaggle “Electric Motor Temperature” dataset. The team plans to use K-Nearest Neighbours (KNN) for temperature prediction.

The system has to answer five questions about a running motor:

- What will the motor temperature be in the next seconds and minutes?

- How long until it reaches a chosen temperature, or starts to overheat?

- At what time should cooling be started?

- For how long should cooling run?

- What will the final temperature be after that cooling?

Key finding for the KNN plan. KNN is a reasonable choice for questions 1 and 2. For questions 3 to 5, KNN alone is not reliable, because it cannot be told that “colder coolant means less heating”. In a test on synthetic data it predicted a hotter motor when the coolant was made colder. Section 4.2 explains this and proposes a hybrid design.

# 2. Dataset summary

The dataset comes from a test bench for a permanent-magnet synchronous motor (PMSM). Each recording session is called a profile. Rows are sampled at 2 Hz, so one row is 0.5 seconds. The full file has about 1.3 million rows across 52 profiles. The copy used in this project shows temperatures in °C (for example, pm of about 24.5 at the start), which should be confirmed before choosing temperature limits.

| Column | Meaning | Role in this project |
| --- | --- | --- |
| pm | Permanent-magnet (rotor) temperature | Primary temperature target |
| stator_winding, stator_tooth, stator_yoke | Stator temperatures | Alternative temperature targets |
| coolant | Coolant temperature | Input; also the lever used to simulate cooling |
| ambient | Ambient temperature | Input |
| u_d, u_q | Voltage in the d and q axes | Inputs (electrical load) |
| i_d, i_q | Current in the d and q axes | Inputs (electrical load) |
| motor_speed, torque | Shaft speed and torque | Inputs (mechanical load) |
| profile_id | Recording session number | Grouping key for train/test splits, never a feature |

# 3. Target variables

Only the first target is a column that can be learned directly from labelled data. The other four are derived by simulating the first one forward in time. This matters for how each is checked.

| ID | Target | Type | Unit | How it is obtained | How it is evaluated |
| --- | --- | --- | --- | --- | --- |
| T1 | Motor temperature (default: pm) | Regression on labelled data | °C | KNN predicts the temperature change over the next 5 s. Repeating this step gives the curve up to 30 min. | MAE, RMSE, R²; multi-step forecast error at 1, 5, 10 and 30 min |
| T2 | Time to reach a chosen temperature (overheating onset) | Derived from T1 | seconds | First step at which the forecast reaches the limit, times 5 s. | Error vs actual crossing time on held-out profiles |
| T3 | Cooling start time | Decision, found by simulation | seconds from now | Latest start that still keeps the peak below the limit, minus a safety margin. | Simulation consistency; no ground-truth labels in the data |
| T4 | Cooling duration | Decision, found by simulation | seconds | Shortest duration that keeps the peak below the limit when started at T3. | Same as T3 |
| T5 | Final temperature after cooling | Derived from simulation | °C | Peak during the window, temperature when cooling ends, and temperature at the end of the horizon. | Plausibility checks; needs real cooling tests to confirm |

## 3.1 T1: Motor temperature

The model does not predict the absolute temperature. It predicts the change over one step Δ of 5 seconds, from the current temperature and the current operating conditions. Predicting the change is easier than predicting the level, and the level is rebuilt by adding the changes up:

T(k+1) = T(k) + f( T(k), ambient, coolant, u_d, u_q, motor_speed, torque, i_d, i_q )

Here f is the KNN regressor. Repeating this 360 times gives a 30-minute forecast. The load inputs are held at their current values, because the future load is unknown.

## 3.2 T2: Time to a chosen temperature

Given a limit T_limit (for example 90 °C), the time to overheating is read directly from the forecast curve:

time_to_limit = 5 s × (first step k where T(k) ≥ T_limit)

If the curve never reaches the limit within the horizon, the answer is “not reached”, together with the predicted peak. If the motor is already at or above the limit, the answer is “now”.

## 3.3 T3 and T4: When to start cooling and for how long

Cooling is tested by simulation. Each scenario is a pair (start delay, duration). For every pair, the forecast is re-run with the cooling effect applied during that window, and the peak temperature is recorded. The recommendation is:

- Cooling duration (T4): the shortest duration whose peak stays below the limit.

- Latest safe start: the latest start delay for which this duration still keeps the peak below the limit.

- Recommended start (T3): the latest safe start minus a safety margin (for example 60 s), so the plan is not at its edge.

If no duration keeps the peak under the limit, the correct output is “cooling alone is not enough; reduce the load as well”. A system that always returns a cooling plan would be misleading.

## 3.4 T5: Final temperature after cooling

Three numbers are reported from the simulated cooled curve: the peak during the horizon, the temperature at the moment cooling stops, and the temperature at the end of the horizon. The last one shows whether the motor starts heating again after cooling stops, which is common when the load stays high.

Illustrative output format (not from the real dataset): “Limit 90 °C reached in 15 min. Start cooling within 14 min and run it for at least 12 min. Peak stays below 90 °C; temperature at the end of 30 min is 88 °C.”

# 4. Algorithms

## 4.1 Overall pipeline

- Prepare data: keep rows in time order within each profile, thin to one row every 5 s (1.3 million rows becomes about 130,000), and drop rows without a next-step label.

- Train KNN (T1): standardise the inputs, then predict the 5-second temperature change.

- Forecast: repeat the one-step prediction to build the 30-minute curve.

- Time to limit (T2): find the first step where the curve reaches the limit.

- Cooling what-if (T3 to T5): simulate many (start, duration) scenarios with a cooling-effect module, then pick the plan.

- Validate: hold out whole profiles, never random rows.

## 4.2 KNN for temperature prediction

KNN finds the k past moments that look most like the current one (same temperature, coolant, ambient, voltages, currents, speed and torque) and averages the temperature change that followed them. It has no training step beyond storing the data, it handles non-linear behaviour, and its predictions are easy to explain by pointing at the neighbours.

| Setting | Recommendation |
| --- | --- |
| Feature scaling | Mandatory. Use StandardScaler. Without it, large-valued inputs such as motor_speed dominate the distance. |
| Target | Temperature change over 5 s, not the absolute temperature. |
| k (neighbours) | Search 5, 15, 30, 60. Small k follows the data closely but is noisy; large k is smooth but lags real changes. |
| Weights | Compare uniform and distance-weighted averaging. |
| Search method | KD-tree on the thinned data. The full 1.3 million rows makes every prediction slow, especially in a 360-step forecast. |
| Validation | GroupKFold by profile_id. A random split lets neighbours from the same profile leak into the test set and makes the score look far better than it is. |

Known limits of KNN for this problem:

- No extrapolation. KNN can only return values seen in the training data. It cannot predict a temperature higher than the data contains, and its response flattens outside the training range.

- No physical constraints. The model cannot be told that colder coolant should reduce heating. Coolant temperature itself rises with the motor temperature in the data, so KNN can learn the wrong direction. In a test on synthetic data, the forecast peak rose from 244 °C to 313 °C when the coolant was lowered by 10 °C. This is a check on synthetic data only, and it must be repeated on the real file (see the sanity check in Appendix A).

- Error build-up. Each step feeds on the previous prediction, so small errors grow over a 30-minute forecast. Report error at several horizons, not only for one step.

- Sensitivity to unhelpful inputs. Every column counts equally in the distance. Feature selection or weighting may be needed.

Recommended design: a hybrid. Use KNN for T1 and T2, where the question is “what happens if conditions stay as they are”. For the cooling effect used in T3 to T5, use a separate module that is constrained to behave physically, either (a) a gradient-boosting model with a monotonic constraint on coolant, as in the prototype script, or (b) a simple heat-transfer correction fitted to the data. The cooling module should be applied as a correction on top of the KNN baseline. If the sanity check in Appendix A passes on the real data, KNN could be used for cooling as well, but this should be shown, not assumed.

## 4.3 Algorithm comparison

| Algorithm | Strengths | Weaknesses | Role |
| --- | --- | --- | --- |
| KNN regression | Simple, non-linear, explainable, no training | No extrapolation, no physical constraints, slow on large data | Primary model for T1 and T2 (as planned) |
| Gradient boosting (HistGradientBoosting) | Accurate, fast, supports a monotonic constraint on coolant | Less explainable than KNN; also cannot extrapolate | Benchmark for T1; cooling-effect module for T3 to T5 |
| Ridge / linear regression | Very fast, easy to interpret | Misses non-linear behaviour | Baseline to beat |
| LSTM / GRU neural network | Learns time patterns directly | Needs more data, tuning and compute | Optional, only if the above are not accurate enough |
| Lumped-parameter thermal model | Physically meaningful cooling response | Needs parameter fitting | Reference for the cooling effect |

## 4.4 Time to limit and cooling planner

Both use recursive simulation, not a separate model. This keeps T2 to T5 consistent with T1: if the temperature forecast improves, all downstream answers improve with it. The alternative, training a model to predict time-to-limit directly, needs a fixed limit at training time and gives no way to test cooling scenarios, so it is not recommended.

Planner steps:

- Forecast with no cooling. If the limit is never reached, stop: no action is needed.

- For every duration from 0 to the horizon (cooling starting now), simulate and record the peak. Take the first duration with peak below the limit.

- Find the latest start delay at which that duration still works.

- Subtract the safety margin to get the recommended start. Re-simulate from that start to get the final duration and the T5 values.

- If nothing works, report that cooling alone is not sufficient.

# 5. Validation plan

All scores are measured on profiles that the model has not seen. The table below lists what will be reported. The last column is to be filled in after running on the Kaggle data.

| Target | Metric | Held-out set | Result (to fill) |
| --- | --- | --- | --- |
| T1 | MAE and RMSE of the 5 s change; R²; compare against “no change” baseline | Held-out profiles |  |
| T1 multi-step | Forecast MAE and worst error at 1, 5, 10 and 30 min, using real future load | Held-out profiles |  |
| T2 | Error in seconds between predicted and actual limit-crossing time; number of missed or false crossings | Held-out profiles that cross the limit |  |
| T3, T4, T5 | Sanity check: lowering coolant never raises the peak; longer cooling never gives a higher peak | Simulation |  |

T3 to T5 have no labelled answers in this dataset, because the data does not record deliberate cooling actions. Their reliability therefore rests on the accuracy of T1 and on the cooling-effect module, and the final numbers should be treated as estimates until compared with real cooling tests.

# 6. Assumptions and limitations

- Coolant is measured, not controlled. In this dataset coolant is a recorded temperature, not an input an operator sets. The simulated cooling effect is an estimate. It should be calibrated against your real cooling system.

- Constant load. Forecasts hold the load at its current value. A load spike makes the real temperature rise faster than predicted.

- Test-bench data. The data comes from a lab test bench. Accuracy on a different motor, cooling circuit or sensor set-up is not guaranteed.

- Temperature units. Confirm that the data is in °C before choosing limits. Public copies of the dataset are described as normalised.

- Thinning. Using every 10th row loses fast dynamics (anything faster than about 5 seconds). This is acceptable for thermal behaviour, which is slow.

- Horizon. Forecasts are limited to 30 minutes. Accuracy beyond that is not assessed.

# 7. Status and next steps

A prototype script exists that trains a gradient-boosting model, forecasts temperature, finds the time to limit, plans cooling and draws a dashboard. It has been tested end to end on synthetic data with the same columns. It has not yet been run on the real Kaggle file, so no accuracy figures for the real data are reported here.

- Rebuild the thinned CSV from the original file, keeping the real profile_id column.

- Train KNN and gradient boosting with the same profile-wise split and fill in the validation table in Section 5.

- Run the coolant sanity check from Appendix A on the real data. If KNN fails it, keep the hybrid design.

- Choose the real temperature limit and the cooling strength for the target system.

- If real cooling-test data becomes available, calibrate and validate T3 to T5 against it.